Implementation of different types of LAN topology with IP addressing using PT

Objectives:

- Design the following topology in Packet Tracer.

- Subnet each of the network with the IP and complete the following table.

- Assign the appropriate IP into each of the network.

- Add one more PC to each of router and assign IP.

- Test connectivity between PC in a router.

Address Space

192.168.1.0 / 24

First subnet (80 hosts)

192.168.1.0 /24

Required hosts 80= 27 =128 hosts

Subnet mask= 11111111. 11111111.11111111. 10000000 =255.255.255.128

N.A= 192.168.1.0 /25      -------                 B.A= 192.168.1.127 /25

Second subnet (40 hosts)

192.168.1.128 /26

Required hosts 40= 26 =64 hosts

Subnet mask= 11111111. 11111111.11111111. 11000000 =255.255.255.192

N.A= 192.168.1.128 /26      --------    B.A= 192.168.1.191 /26

3rd subnet (20 hosts)

192.168.1.192 /27

Required hosts 20= 25 =32 hosts

Subnet mask= 11111111. 11111111.11111111. 11100000 =255.255.255.224

N.A= 192.168.1.192 /27 -----------   B.A= 192.168.1.223 /27

4th  subnet (2 hosts)

192.168.1.224 /30

Required hosts 20= 22 =4 hosts

Subnet mask= 11111111. 11111111.11111111. 11111100 =255.255.255.252

N.A= 192.168.1.224 /30 ----------- B.A= 192.168.1.227/30

5th  subnet (2 hosts)

192.168.1.228 /30

Required hosts 20= 22 =4 hosts

Subnet mask= 11111111. 11111111.11111111. 11111100 =255.255.255.252

N.A= 192.168.1.228 /30   -----------  B.A= 192.168.1.231/30

Subnet Table

| Subnet | Assignable Adress Range | Subnet Mask | Subnet Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| First subnet (80 hosts) | 192.168.1.1 /25 -192.168.1.126 /25 | 255.255.255.128 | 192.168.1.0 /25 | 192.168.1.127 /25 |
| Second subnet (40 hosts) | 192.168.1.129 /26 - 192.168.1.190 /26 | 255.255.255.192 | 192.168.1.128 /26 | 192.168.1.191 /26 |
| 3rd subnet (20 hosts) | 192.168.1.193 /27- 192.168.1.222 /27 | 255.255.255.224 | 192.168.1.192 /27 | 192.168.1.223 /27 |
| 4th subnet (2 hosts) | 192.168.1.225 /30- 192.168.1.226/30 | 255.255.255.252 | 192.168.1.224 /30 | 192.168.1.227/30 |
| 5th subnet (2 hosts) | 192.168.1.229/30- 192.168.1.230/30 | 255.255.255.252 | 192.168.1.228 /30 | 192.168.1.231 /30 |

Addressing table

| Device | Interface | IP address | Subnet mask | Default Gateway |
| --- | --- | --- | --- | --- |
| HQ | Fast ethernet 0/0 | 192.168.1.190 /26 | 255.255.255.192 | N/A |
|  | Serial 2/0 | 192.168.1.225 /30 | 255.255.255.252 |  |
|  | Serial 3/0 | 192.168.1.229/30 | 255.255.255.252 |  |
| B1 | Fast ethernet 0/0 | 192.168.1.126 /25 | 255.255.255.128 |  |
|  | Serial 2/0 | 192.168.1.226/30 | 255.255.255.252 |  |
| B2 | Fast ethernet 0/0 | 192.168.1.222 /27 | 255.255.255.224 |  |
|  | Serial 3/0 | 192.168.1.230/30 | 255.255.255.252 |  |
| PC1 | N/A | 192.168.1.1 /25 | 255.255.255.128 | 192.168.1.126 /25 |
| PC2 | N/A | 192.168.1.129 /26 | 255.255.255.192 | 192.168.1.190 /26 |
| PC3 | N/A | 192.168.1.193/27 | 255.255.255.224 | 192.168.1.222 /27 |

Static Routing Table for Router

| Router | Unknown Network Address | Mask Address | Next Hop |
| --- | --- | --- | --- |
| HQ | 192.168.1.0 /25 | 255.255.255.128 | 192.168.1.226/30 |
|  | 192.168.1.192/27 | 255.255.255.224 | 192.168.1.230/30 |
| B1 | 192.168.1.128 /26 | 255.255.255.192 | 192.168.1.225 /30 |
|  | 192.168.1.192 /27 | 255.255.255.224 | 192.168.1.225 /30 |
|  | 192.168.1.228 /30 | 255.255.255.252 | 192.168.1.225 /30 |
| B2 | 192.168.1.128 /26 | 255.255.255.192 | 192.168.1.229/30 |
|  | 192.168.1.0 /25 | 255.255.255.128 | 192.168.1.229/30 |
|  | 192.168.1.224 /30 | 255.255.255.252 | 192.168.1.229/30 |