Implementation of different types of LAN topology with IP addressing using PT

Objectives:

- Design the following topology in Packet Tracer.

- Subnet each of the network with the IP and complete the following table.

- Assign the appropriate IP into each of the network.

- Add one more PC to each of router and assign IP.

Test connectivity between PC in a router.

Address Space

192.168.1.0 / 20

First subnet (6000 hosts)

Required hosts = 6000 ≈   8192 = 2^13  ,Prefix portion = 32 -13 = 19

Subnet mask =  11111111. 11111111.11100000. 00000000 =255.255.224.0

N.A= 192.168.1.0 /19

B.A= 192.168.000000001.00000000

192.168.00011111.11111111

192.168.31.255 /19

Second subnet (3200 hosts)

Required hosts 3200 ≈  4096 = 2^12  ,Prefix portion = 32 -12 = 20

Subnet mask = 11111111. 11111111.11110000. 00000000 =255.255.240.0

N.A  = 192.168.32.0  /20

B.A= 192.168.00100000.00000000

192.168.00101111.11111111

192.168.47.255/20

3rd subnet (300 hosts)

Required hosts 300 ≈  512 = 2^9  ,Prefix portion = 32 -9 = 23

Subnet mask = 11111111. 11111111.11111110. 00000000 =255.255.254.0

N.A  = 192.168.48.0  /23

B.A= 192.168.00110000.00000000

192.168.00110001.11111111

192.168.49.255/23

4th  subnet (100 hosts)

Required hosts 100 ≈  128 = 2^7  ,Prefix portion = 32 -7 = 25

Subnet mask = 11111111. 11111111.11111111. 10000000 =255.255.255.128

N.A  = 192.168.50.0  /25

B.A= 192.168.00110010.00000000

192.168.00110010.01111111

192.168.50.127/25

5th  subnet (80 hosts)

Required hosts 80  ≈ 128 = 2^7  ,Prefix portion = 32 -7 = 25

Subnet mask = 11111111. 11111111.11111111. 10000000 =255.255.255.128

N.A= 192.168.50.128 /25   -----------  B.A= 192.168.50.255/25

6th  subnet (2 hosts)

Required hosts 2 ≈  4 = 2^2  hosts ,Prefix portion = 32 -2 = 30

Subnet mask= 11111111. 11111111.11111111. 11111100 =255.255.255.252

N.A= 192.168.51.0 /30   -----------  B.A= 192.168.51.3/30

7th  subnet (2 hosts)

Required hosts 2 ≈  4 = 2^2  hosts ,Prefix portion = 32 -2 = 30

Subnet mask= 11111111. 11111111.11111111. 11111100 =255.255.255.252

N.A= 192.168.51.4 /30   -----------  B.A= 192.168.51.7/30

Subnet Table

| Subnet | Assignable Adress Range | Subnet Mask | Subnet Address | Broadcast Address |
| --- | --- | --- | --- | --- |
| First subnet (6000 hosts) | 192.168.1.1 /19 -192.168.31.254 /19 | 255.255.224.0 | 192.168.1.0 /19 | 192.168.31.255 /19 |
| Second subnet (3200 hosts) | 192.168.32.1 /20- 192.168.47.254/20 | 255.255.240.0 | 192.168.32.0 /20 | 192.168.47.255/20 |
| 3rd subnet (300 hosts) | 192.168.48.1 /23 - 192.168.49.254 /23 | 255.255.254.0 | 192.168.48.0 /23 | 192.168.49.255/23 |
| 4th subnet (100 hosts) | 192.168.50.1 /25 - 192.168.50.126/25 | 255.255.255.128 | 192.168.50.0 /25 | 192.168.50.127/25 |
| 5th subnet (80 hosts) | 192.168.50.129 /25- 192.168.50.254/25 | 255.255.255.128 | 192.168.50.128 /25 | 192.168.50.255/25 |
| 6th subnet (2 hosts) | 192.168.51.1 /30 – 192.168.51.2/30 | 255.255.255.252 | 192.168.51.0 /30 | 192.168.51.3/30 |
| 7th subnet (2 hosts) | 192.168.51.5 /30 – 192.168.51.6/30 | 255.255.255.252 | 192.168.51.4 /30 | 192.168.51.7/30 |

Addressing table

| Device | Interface | IP address | Subnet mask | Default Gateway |
| --- | --- | --- | --- | --- |
| R3 | Fast ethernet 0/0 |  |  | N/A |
|  | Serial 2/0 |  |  |  |
|  | Serial 3/0 |  |  |  |
| R4 | Fast ethernet 0/0 |  |  |  |
|  | Serial 2/0 |  |  |  |
| R5 | Fast ethernet 0/0 |  |  |  |
|  | Serial 3/0 |  |  |  |
| PC4 | N/A |  |  |  |
| PC5 | N/A |  |  |  |
| PC6 | N/A |  |  |  |
| PC7 | N/A |  |  |  |
| PC8 | N/A |  |  |  |
| PC9 | N/A |  |  |  |